| Group Code.: LHC/VAC |  |
| --- | --- |
| EDMS No.: XXXX |  |
| LHC Project document No.: LHC-VX-ES-0001 |  |

The Large Hadron Collider Project	IT-2926/LHC/LHC

# Invitation to Tender

# Technical Specification for

# Beryllium Beampipes

# EDMS Approval Version 23/6/02

# Abstract

- This Technical Specification concerns the supply of Ultra-High Vacuum chambers for the ALICE, ATLAS and CMS experimental vacuum systems. They are to be manufactured principally from beryllium metal with transitions to aluminium alloy and stainless steel. The longest chamber is 7.3m.

# Table of Contents

1.	INTRODUCTION	1

1.1	Introduction to CERN	1

1.2	Introduction to the LHC Project	1

1.3	Subject of this Technical Specification	1

2.	SCOPE of the tender	1

2.1	Scope of the supply	1

2.2	Items supplied by CERN	2

2.2.1	Items to be loaned for factory testing at the contractor	2

2.2.2	Items to be free-issued by CERN	2

2.3	Long-term conditions	2

3.	general conditions for tendering and contracting	2

3.1	Tender procedure	3

3.1.1	Pre-tender discussions	3

3.1.2	Alternative solutions	3

3.1.3	Preliminary programme	3

3.1.4	Subcontractors	3

3.1.5	Technical Questionnaire	3

3.1.6	Country of origin	3

3.2	Contract execution	3

3.2.1	Responsibility for design, components and performance	3

3.2.2	Contract follow-up	4

3.2.3	Deviations from this Technical Specification	4

3.3	Factory access	4

4.	Technical requirements	4

4.1	General description	4

4.2	Materials	5

4.2.1	Beryllium	5

4.2.2	Aluminium	5

4.2.3	Flanges	5

4.2.4	Stainless steel	5

4.2.5	Copper plating	5

4.3	Dimensions and tolerances	6

4.4	Joints	6

4.5	Safety	6

4.6	Operational conditions	6

4.7	Vacuum requirements	6

4.7.1	Inner (Beam Vacuum) Chambers	6

4.7.2	Outer (Insulation Vacuum) Chamber	7

4.8	Environmental conditions	7

4.9	Witness samples	7

4.10	Heater attachment	8

4.11	Information and documentation management	8

4.11.1	Planning and scheduling	8

4.11.2	Quality control records	8

5.	applicable documents	8

5.1	Standards	8

5.1.1	CERN standards	8

5.1.2	International standards	8

6.	Quality Assurance Provisions	9

7.	tests	9

7.1	Tests to be carried out at the Contractor's premises	9

7.1.1	Organisation of the tests	9

7.1.2	Mechanical tests	9

7.1.3	Vacuum tests	10

7.1.4	Geometrical Tests	10

7.2	Tests to be carried out at CERN	10

8.	delivery and commissioning	10

8.1	Provisional delivery schedule	10

8.2	Packing and transport to CERN	10

8.3	Acceptance and guarantee	10

9.	cern contact PERSONS	11

Annex A:	Components to be supplied by CERN for tests	12

Annex B:	Components to be supplied by CERN for integration	13

Annex C:	Material to be supplied by CERN for integration	14

Annex D:	List of Drawings	15

Annex E:	CD-Rom "CERN Official Documents"	15

# List of Tables

Table 1 - Items to be Supplied	2

Table 2 – LHC QAP topics and documents	9

# Terms and Definitions

| Term | Definition |
| --- | --- |
| CDD | CERN Drawing Directory |
| EDMS | Engineering Data Management System |
| QAP | Quality Assurance Plan |

# INTRODUCTION

## Introduction to CERN

The European Organization for Nuclear Research (CERN) is an intergovernmental organization with 20 Member States*.  It has its seat in Geneva but straddles the Swiss-French border.  Its objective is to provide for collaboration among European States in the field of high energy particle physics research and to this end it designs, constructs and runs the necessary particle accelerators and the associated experimental areas.

At present more than 5000 physicists from research institutes world-wide use the CERN installations for their experiments.

## Introduction to the LHC Project

The Large Hadron Collider (LHC) is the next accelerator being constructed on the CERN site.  The LHC machine will mainly accelerate and collide 7 TeV proton beams but also heavier ions up to lead.  It will be installed in the existing 27 km circumference tunnel, about 100 m underground, that previously housed the Large Electron Positron Collider (LEP).  The LHC design is based on superconducting twin-aperture magnets which operate in a superfluid helium bath at 1.9 K.

In certain areas of the machine, the vacuum chambers containing the circulating beams pass to room temperature zones. Four large particle physics experiments (ALICE, ATLAS, CMS and LHCb) are installed around the machine vacuum chamber in these zones.

## Subject of this Technical Specification

This specification concerns the supply of ultra-high vacuum (UHV) chambers for the LHC experimental vacuum systems. Special requirements in this area for transparency to particles combined with very good vacuum and mechanical performance mean that the chambers must be manufactured from beryllium metal. This implies specialised manufacturing techniques and safety precautions.

# SCOPE of the tender

## Scope of the supply

This supply comprises detailed design, procurement of materials, manufacture, assembly, testing, packaging and delivery to the CERN Prevessin site of three beryllium UHV chambers as outlined in table 1 and their associated documentation.

Table 1 - Items to be Supplied

| Item | Description | Assembly drawing number | Design Temperature |
| --- | --- | --- | --- |
| 1 | ALICE beampipe | LHC VC2C_0001 | 300C |
| 2a | ATLAS, single walled beampipe with attached heaters | LHC VC1I_0009 | 250C |
| 2b | ATLAS double walled beampipe: As 2a, but with second concentric wall, assembled with spacers, bellows and pump-out ports. | LHC VC1I_0003 | 250C |
| 3 | CMS beampipe | LHC VC5C_0001 | 300C |

As a result of the tender, CERN will decide between variant 2a or 2b for the ATLAS experiment.

## Items supplied by CERN

### Items to be loaned for factory testing at the contractor

Flanges, transitions and seals described in Annex A will be supplied to the contractor for the duration of factory testing, and shall be returned to CERN with the final delivery.

### Items to be free-issued by CERN

CERN will supply components and quantities listed in Annex B for incorporation into the relevant items.

CERN will also supply the raw materials and quantities listed in Annex C for incorporation into the finished products. If the supplier wishes to use different materials for the non-beryllium components of the beampipes, they shall be proposed as part of the detailed design.

## Long-term conditions

In case CERN no longer requires the beryllium beampipes, the contractor undertakes to ensure safe disposal of the beryllium free of charge to CERN for the period of 10 years following the placement of the contract.

# general conditions for tendering and contracting

Please refer to the commercial documents for more complete information.

Tenders will only be considered from firms having been selected as qualified bidders by CERN, as a result of the Market Survey MS-2926/LHC/LHC.  CERN reserves the right to disqualify any bidder whose reply to this Market Survey is found to have been incorrect.

## Tender procedure

### Pre-tender discussions

The Bidder is strongly encouraged to contact CERN and discuss details of this Technical Specification before submitting a tender.  In particular, CERN wishes to ensure that no doubt exists as to the interpretation of this Technical Specification.

### Alternative solutions

If the Bidder finds that any part of this Technical Specification is difficult, or costly to meet, he is free to propose an alternative solution, provided that the deviations from this Technical Specification, together with the reasons and advantages, are clearly indicated in the tender.  Such alternative solutions shall always be made in addition to a conforming bid, which must comply fully with this Technical Specification.

CERN reserves the right to accept or reject the proposed alternative solutions without justification.

### Preliminary programme

The Bidder shall propose a preliminary design and manufacturing schedule with the tender, based on the specified CERN provisional delivery schedule.

### Subcontractors

The Bidder shall declare in his Tender any subcontractors whose services he intends to use in the event of a Contract.  Refer to the commercial documents for more details. If awarded the Contract, the Bidder shall restrict himself both to the subcontractors and the amount mentioned in the Tender.  If, for some reason, he wants to change any subcontractor, or the scope of subcontracted work, or the amount subcontracted, he must obtain CERN’s prior agreement in writing.

### Technical Questionnaire

The Technical Questionnaire attached to this Technical Specification shall be completely filled in and returned with the Tender Form, otherwise the tender will not be considered as complete and will be discarded.

### Country of origin

Please refer to the commercial documents for specific conditions concerning the country of origin of the equipment or services to be supplied.

## Contract execution

### Responsibility for design, components and performance

The Contractor shall be responsible for the correct performance of all items supplied, irrespective of whether they have been chosen by the Contractor or suggested by CERN.  CERN's approval of the design and component choice does not release the Contractor from his responsibilities in this respect.

CERN assumes responsibility for the performance of items and sub-systems supplied by CERN.

### Contract follow-up

#### Contract engineer

The Contractor shall assign an engineer to be responsible for the technical execution of the Contract and its follow-up throughout the duration of the Contract.

#### Progress report

The Contractor shall supply, within one month of notification of the Contract, a written programme detailing the manufacturing and testing schedules.  The programme shall include preliminary dates for inspections and tests.

A written progress report shall be sent to CERN every month until completion of the Contract.

#### Design approval and production

The detailed design shall be submitted to CERN for approval within two months after notification of the contract. CERN will give its approval or refusal, in writing, within three weeks.  Component ordering and equipment manufacture shall not start without CERN’s written prior agreement.

The series assembly shall be preceded by the production of witness samples described in section 4.9. Assembly of the series shall not start before CERN has given its formal approval of the witness samples in writing.

### Deviations from this Technical Specification

If, after the Contract is placed, the Contractor discovers that he has misinterpreted this Technical Specification, this will not be accepted as an excuse for deviation from it and the Contractor shall deliver equipment in conformity with this Technical Specification at no extra cost.

During execution of the Contract, all deviations proposed by the Contractor from this Technical Specification, the Tender, or any other subsequent contractual agreement, shall be submitted to CERN in writing.  CERN reserves the right to reject or accept such proposals without justification.

CERN reserves the right to modify this Technical Specification during execution of the Contract.  The consequences of such modifications shall be mutually agreed between CERN and the Contractor.

## Factory access

CERN and its representatives shall have free access during normal working hours to the manufacturing or assembly sites, including any subcontractor’s premises, during the Contract period.  The place of manufacture, as stated in the Tender, may only be changed after written approval by CERN.

# Technical requirements

## General description

Assembly drawings for the beampipes detailing CERN’s requirements are listed in Table 1. Component drawings referred to in these assemblies are listed in Annex D.  The contractor shall make a detailed design to conform with these drawings and the requirements of this specification. The contractor shall then produce manufacturing drawings for approval as per section 3.2.2.3.

The chamber shall be assembled from beryllium tubes with a minimum length of 950mm per tube, except at each extremity, where shorter sections are permitted to obtain the required length.

## Materials

### Beryllium

Beryllium tubes shall be manufactured by machining from blocks of solid beryllium rod conforming to Brush-Wellman specification S-200F. Documentary proof of conformity to this specification must be provided for all beryllium used. The Contractor shall perform non-destructive testing (NDT) to ensure that the machined tubes will comply with the mechanical and vacuum requirements. The proposed NDT programme shall form part of the detailed design and be submitted to CERN for approval as per section 3.2.2.3. External beryllium surfaces of the chamber shall be coated with a Berylcoat D or equivalent protective coating.

### Aluminium

Aluminium sections shall be machined from forged blocks of AA2219 in the T-6 or T-8511 state. The aluminium flange body shall not be heated above 300C to avoid permanent loss of mechanical properties.

### Flanges

Flanges are minimised in dimension and designed for use with Helicoflex seals. The flange details shall conform to drawings LHC VFX__0002, LHC VFX__ 0001 and LHC VC5C_ 0002. The surface finish of the sealing surface shall be ISO 1302 N7C or N6C (0.8m<Ra1.6m). For technical reasons, a surface finish of N5 or N8 is unacceptable. The sealing surfaces shall be without scratches, marks or other damage.

### Stainless steel

Stainless steel tubes and cones shall be machined from AISI 316L. If assembly of stainless steel by electron beam welding is foreseen, this must be specified in the detailed design and care must be taken to ensure that the material is suitable.  Flanges shall be machined from AISI 316LN forged blanks according to CERN specification No.1001 Edition 3.

### Copper plating

Stainless steel sections of items 1 and 3 of Table 1 shall be electroplated with 100m  20 m of copper. The copper shall be of high purity (>99.95%) and no organic additives shall be present in the plating bath. The relative magnetic permeability of the resulting chamber after coating shall not exceed 1.02. This implies that care must be taken with any strike applied. The proposed strike and copper plating procedure shall be approved in writing by CERN.

The copper layer shall be such that it is mechanically inseparable from the stainless steel when using a screwdriver or similar tool. No blisters shall appear after 20 thermal cycles to 300C. The contractor shall make samples TBD to verify these requirements.

## Dimensions and tolerances

Drawings are dimensioned according to ISO standards. Due to the non-rigid nature of the final assemblies, ISO 10579-NR is applied. The jigs and support conditions required to measure the tolerances are defined in the drawings to ensure that tolerances are respected under operating support conditions.

## Joints

Leak tight joints between beryllium-beryllium, beryllium-aluminium and beryllium-stainless steel are critical for this application. The joint methods proposed shall be specified in the attached technical questionnaire. They shall not be changed without written authorization from CERN. The detailed design (see section 3.2.2.3) shall include joint geometry and all materials to be used.

## Safety

CERN Safety Instruction No. 25 “Beryllium” applies. In particular, there must be no contamination of surface with beryllium dust.  In this context, contamination is defined as more than 0.1 mg/m2.  Any tube not conforming will be returned for decontamination at the Contractor's charge.

It should be noted that for reasons of safety, no machining of beryllium (including the use of hand tools, abrasive paper etc.) is allowed on the CERN site.

## Operational conditions

The beampipes shall be designed to withstand all of the following load combinations:

- vacuum conditioning loads: temperature as per table 1. Simply supported at flanges under own weight, differential external pressure of 1 atmosphere, 50 load cycles

- overpressure test loads:  temperature 20°C. Simply supported at flanges under own weight, differential external pressure of 1.5 atmospheres, 50 load cycles

- operation loads: temperature -20°C (negative 20°C). Simply supported at flanges under own weight, external differential pressure of 1 atmosphere, 5000 load cycles

- transport loads: temperature 20°C, no differential pressure. Uniform axial bending moment of 50 Nm, 100 load cycles.

## Vacuum requirements

### Inner (Beam Vacuum) Chambers

The required base pressure of the beam vacuum chambers is about 10-9 Pa (10-11 Torr).  In order to achieve such ultra-high vacuum, cleanliness of the inner surface at all stages of assembly work is very important.

The delivered vacuum chamber shall be clean with no further cleaning required at CERN.  This implies in particular no traces of hydrocarbons, organic or inorganic residues from the fabrication or the handling and no finger prints on the inside surface. The proposed cleaning procedure shall be included in the detailed design.  Adequate protections of the ends of the tubes shall be provided at all times to maintain the state of cleanliness.  Sealing materials containing silicon grease cannot be used.

The surfaces shall be free from visible porosity, which may give rise to virtual leaks.

The vacuum chamber shall be leak-tight by UHV standards when tested with a global external pressure of one bar of helium.  Leak tightness of the chamber shall be defined as a total leak rate measured on a calibrated He leak detector not exceeding 2 x 10-11 Pa m s-1 (1.5 10-10 Torr l s-1).

The inner total surface of the vacuum chamber shall have a total outgassing rate not exceeding 4 x 10-9 Pa m3 s-1 m-2 (3 x 10-12 Torr l s-1 cm-2) 72 hours after a 24-hour bakeout at 250°C.  The outgassing rate of hydrocarbons shall not exceed 1% of the total.

### Outer (Insulation Vacuum) Chamber

The ATLAS double walled variant (item 2b) has a second concentric beryllium wall, which contains an insulation vacuum, in order to minimise heat transfer to nearby components during bakeout. This insulation vacuum volume is maintained at a pressure down to 2 x 10-4 Pa  (1.5 10-6 Torr).

The insulation vacuum volume shall be clean with no further cleaning required at CERN.  Care shall be taken to ensure that all components installed in the volume are clean and free of grease. All assembly processes shall be performed with clean lint-free gloves.  Adequate protections on the pumping ports and extremities shall be provided at all times.  Sealing materials containing silicon grease cannot be used.

The insulation vacuum chamber shall be leak-tight by UHV standards when tested with a global external pressure of one bar of helium.  Leak tightness of the chamber shall be defined as a total leak rate measured on a calibrated He leak detector not exceeding 2 x 10-11 Pa m s-1 (1.5 10-10 Torr l s-1).

## Environmental conditions

The vacuum chamber will operate in a highly radioactive environment (~107 Gy integrated dose over the design lifetime). No materials shall be used that cannot withstand this environment.

## Witness samples

Three witness samples shall be supplied to CERN for approval for each joint geometry or material pair. In each case the sample shall consist of tubes of material from the same batch as those to be joined in the final assemblies, and with the same geometry. They shall be joined together using the same parameters, materials and in the same equipment as the final assemblies.

Two samples for each pair shall then be subjected to a destructive traction test by the contractor, with stress-strain curves recorded and sent to CERN, along with the failure loads and broken samples. In case of problem with the test sample or equipment, the contractor shall repeat the tests at his expense with new samples such that two tests are completed and documented.  The third sample shall be sectioned across the joint, potted in resin, prepared for microscopic examination in a manner TBD and sent to CERN.

## Heater attachment

The ATLAS beampipe (items 2a or 2b in Table 1) require heaters to be attached to the chamber.  The heaters and consumable materials required for assembly will be supplied by CERN as per Annex C. All tooling required for attachment shall be supplied by the contractor. In particular, this process requires a vacuum chamber (operating at ~10-4 Pa) able to contain the whole beampipe. The heaters shall be attached following a procedure TBD by CERN.

## Information and documentation management

### Planning and scheduling

Planning and scheduling activities shall be performed according to the procedure defined in the LHC QAP document No LHC-PM-QA-301.01, "Planning and Scheduling Requirements for Institutes, Contractors and Suppliers".

### Quality control records

All specified tests and measurements carried out during all stages of production, from raw material procurement up to delivery and installation, must be recorded in specific files, collected in the MTF (Manufacturing and Test Folder), according to the procedure defined in the LHC QAP document No LHC-PM-QA-309.00, "Fabrication and Inspection of Purchased Equipment".

# applicable documents

Please refer to the cover letter or Instructions to Bidders for the complete list of enclosed documents which form part of this Invitation to Tender.

Please note that the quality assurance documents, CERN standards and Purchasing documents referred to in this Technical Specification are on the enclosed CD-Rom entitled "CERN Official Documents".

## Standards

The following additional standards are applicable for the execution of the Contract.

### CERN standards

- Safety Instruction No. 25 “Beryllium”

- Material Technical Specification No.1001, edition 3. “Forged Blanks for Vacuum Applications”

### International standards

- ISO 1302:2002 “Geometrical Product Specifications (GPS) -- Indication of surface texture in technical product documentation”

- ISO 10579:1993 “Technical drawings -- Dimensioning and tolerancing -- Non-rigid parts”

# Quality Assurance Provisions

The Contractor must plan, establish, implement and adhere to a documented quality assurance program that fulfils all the requirements described in this Technical Specification and drawn up according to the Quality Assurance Plan for the LHC Project.

Please note that the quality assurance documents, CERN standards and Purchasing documents referred to in this Technical Specification are on the enclosed CD-Rom entitled "CERN Official Documents".

The list of relevant topics covered by the LHC Quality Assurance Plan, together with the corresponding documents, is given in table 2 below.

Table 2 – LHC QAP topics and documents

| - Topic | - Document Title | - Doc. Number |
| --- | --- | --- |
| - Policy and Organisation | - Quality Assurance Policy and Organisation | - LHC-PM-QA-100.00 |
|  | - Glossary, Acronyms, Abbreviations | - LHC-PM-QA-203.00 |
| - Planning | - Planning and Scheduling Requirements for Institutes, Contractors and Suppliers | - LHC-PM-QA-301.01 |
| - Design | - Quality Assurance Categories | - LHC-PM-QA-201.00 |
|  | - Design Process and Control | - LHC-PM-QA-307.00 |
|  | - Drawing Management and Control | - LHC-PM-QA-305.00 |
| - Change Control | - Configuration Management - Change Process And Control | - LHC-PM-QA-304.00 |
| - Manufacturing and Inspection | - Manufacturing and Inspection of Equipment | - LHC-PM-QA-309.00 |
|  | - Handling of Non-conforming Equipment | - LHC-PM-QA-310.00 |
|  | - LHC Part Identification | - LHC-PM-QA-206.00 |

# tests

## Tests to be carried out at the Contractor's premises

### Organisation of the tests

CERN reserves the right to be present, or to be represented by an organization of its choice, to witness any tests carried out at the Contractor's or his subcontractors' premises.  The Contractor shall give at least 15 working days notice of the proposed date of any such tests. The Contractor shall provide all test equipment and personnel necessary to perform the tests required in this specification, except for that equipment noted in section 2.2.1.

### Mechanical tests

The vacuum chambers shall each be subjected to the following tests in sequence:

- 24 hours at temperature as per Table 1 with an applied differential external pressure of 1 bar and simply supported at flanges under own weight

- 2 hours at 20C with an applied differential external pressure of 1.5 bar and Simply supported at flanges under own weight

- 24 hours at temperature as per table 1 with an applied differential external pressure of 1 bar and simply supported at flanges under own weight

The ATLAS double-walled chamber (item 2b from Table 1) shall undergo vacuum and mechanical tests twice during the assembly. Firstly as a single-walled tube, as per drawing LHC VC1I_0009 and secondly as a completed assembly as per drawing LHC VC1I_0003. In both cases, the chamber shall be heated using the installed bakeout heaters.

### Vacuum tests

The leak rate requirements of section 4.7 shall be verified at room temperature before and after mechanical tests with one bar external pressure of helium for one hour. Testing shall be performed with a Helium leak detector of sensitivity better than 2 x 10-11 Pa m s-1 (1.5 10-10 Torr l s-1), calibrated immediately before the operation.

### Geometrical Tests

Tolerances on the final assembly shall be verified using the inspection jigs defined in drawings LHC VC5C_0001, LHC VC1I_0010 and LHC VC2C_0002.

## Tests to be carried out at CERN

The leak rate, limit pressure and outgassing requirements of section 4.7 will be verified at CERN after delivery. If the vacuum chamber fails to meet these requirements it will be returned by CERN to the Contractor for cleaning or repair and returned to CERN, with no additional cost to CERN.

# delivery and commissioning

## Provisional delivery schedule

The delivery of the final chamber should take place 12 months after placement of the contract.  The precise sequence of delivery is TBD between CERN and the contractor.

## Packing and transport to CERN

The Contractor is responsible for the packing and, where included, the transport to CERN.  He shall ensure that the equipment is delivered to CERN without damage and any possible deterioration in performance due to transport conditions.

## Acceptance and guarantee

Provisional acceptance will be given by CERN only after all items have been delivered in accordance with the conditions of the contract including documentation referred to in this Technical Specification, all tests specified have been successfully completed and all test or other certificates have been supplied to CERN.

The guarantee period is defined in the commercial documents.

# cern contact PERSONS

Persons to be contacted for technical matters:

| Name/Division/Group | Tel-Fax | Email |
| --- | --- | --- |
| Raymond VENESS LHC/VAC In case of absence: Patrick LEPEULE LHC/VAC | Tel: +41 22 767 3009 Fax: +41 22 767 5100 Tel: +41 22 767 4352 Fax: +41 22 767 5100 | Raymond.Veness@cern.ch Patrick.Lepeule@cern.ch |

- Components to be supplied by CERN for tests

| Description | Drawing Number | Number supplied |
| --- | --- | --- |
| Blank flange OD 86mm | LHC VFX_P 0008.A | 4 |
| Transition OD 86mm to Conflat OD 114mm | LHC VFX_P 0005.A | 1 |
| Blank flange OD 130mm | TBD | 2 |
| Transition OD 130mm to Conflat 114mm | TBD | 1 |
| Helicoflex seals for OD 86mm flange | - | 8 |
| Helicoflex seals for OD 130mm flange | - | 4 |

- Components to be supplied by CERN for integration

| Item | Component | Drawing | Quantity |
| --- | --- | --- | --- |
| 1. ALICE | Bellows | TBD | 3 |
| 2a. ATLAS single wall | Bakeout heaters, 3.5m long | - | 3 |
|  | Self polymerising Kapton tape (roll) | - | 1 |
|  | Thermal shrinking tape (roll) | - | 1 |
| 2b. ATLAS Double wall | Spacers | TBD | 4 |
|  | Aluminium bellows | LHC VC1I_0008 | 1 |
|  | Aluminised Kapton foil (roll) |  | 1 |

- Material to be supplied by CERN for integration

| Material | Dimensions | Quantity |
| --- | --- | --- |
| AA 2219 T-6 Forging | 90mm diameter x 60mm | 3 |
| AA 2219 T-8511 Extruded bar | 72.5mm diameter x 300mm | 2 |
| AISI 316L Hollow jet | 118 OD x 63 ID x 1000 mm | 4 |
| AISI 316L Hollow jet | 71 OD x 36 ID x 400 mm | 3 |
| AISI 316L Forged Blank | 152 mm diameter x 23 mm | 3 |
| 316LN Forged bar | 120mm x 110 mm | 1 |
| 316LN Forged blank flange | DN 63 / OD 113 mm | 2 |

- List of Drawings

| Item | Drawing | Title |
| --- | --- | --- |
| 1. ALICE | LHC VC2C_0001 | Central Part |
|  | LHC VC2C_0002 | Alignment Jig |
|  | LHC VFX__0002 | Minimised Fixed Flange |
|  | STD VFUHV 0007 | Conflat Flange |
| 2a. ATLAS Single wall | LHC VC1I_0009 | Inner Tube |
|  | LHC VFX__ 0001 | Minimised Fixed Flange |
|  | LHC VC1I_0010 | Alignment of Inspection Jig |
| 2b ATLAS Double wall | LHC VC1I_0003 | Inner Detector (VI) |
|  | LHC VC1I_0011 | Outer Tube |
|  | LHC VC1I_0008 | Bellows (with flange) |
| 3. CMS | LHC VC5C_0001 | Central Beampipe |
|  | LHC VC5C_0002 | Beryllium Tube |
|  | LHC VC5C_0003 | Stainless Steel Cone |

- CD-Rom "CERN Official Documents"

*	CERN Member States are: Austria, Belgium, Bulgaria, Czech Republic, Denmark, Finland, France, Germany, Greece, Hungary, Italy, The Netherlands, Norway, Poland, Portugal, Slovak Republic, Spain, Sweden, Switzerland and the United Kingdom.