# LWFA Micro-valve and Nozzle Requirements Definition Document

### Science Requirement:

A valve and nozzle are required to create a variable density gas jet.  Pulsed lasers will ignite a plasma channel in the jet that will be used to explore a parameter regime for Laser Wake Field Acceleration (LWFA) research at LBNL.

### Engineering Requirements Table

| ## Index | ### Requirement | Value | Weighting Factor (1-5, lowest - highest) | Requirement Type | Notes |
| --- | --- | --- | --- | --- | --- |
| 1.0 | ### Electron density |  |  | Functional | Depending on working gas type, atmospheric density is commensurately lower. |
| 1.1 | Minimum density | 1017 /cm3 | 2 | Functional |  |
| 1.2 | Nominal Density | 1018 /cm3 | 5 |  |  |
| 1.3 | Maximum density | 1021 /cm3 | 3 | Functional |  |
| 2.0 | ### Density Gradient | 100 m | 4 | Functional | At location of laser interaction (i.e. 1 mm from nozzle) |
| 3.0 | Density Fluctuation | <5 % | 4 | Functional | This is the fluctuation with respect to the nominal denisity, which may vary along the beam axis |
| 4.0 | ### Jet Length |  |  | Functional |  |
| 4.1 | Min | 1 cm | 5 |  |  |
| 4.2 | Max | 10 cm | 2 |  |  |
| 5.0 | ### Jet width |  |  | Functional |  |
| 5.1 | Min | 100 m | 4 |  |  |
| 5.2 | Max | 300 m | 5 |  |  |
| 6.0 | ### Duration of gas jet pulse |  |  | Functional | Depends on pumping speed |
| 6.1 | Min | 10 s | 2 |  |  |
| 6.2 | Max | 1 ms | 4 |  |  |
| 7.0 | ### Gas Types |  |  | Functional |  |
| 7.1 | He | - | 5 |  | Currently using He |
| 7.2 | Ag | - | 1 |  |  |
| 8.0 | Shaped density profile | yes | 5 | Functional | Control injection and acceleration of gas/fluid along length of jet |
| 9.0 | Parametric control |  |  | Functional | Adjust valve and/or nozzle to change the jet |
| 9.1 | Multiple distributions with one system | yes | 5 |  | No need to open up the vacuum system to swap out valve or nozzle hardware |
| 9.2 | Variability of density within jet | 100 % | 3 |  | Depends on linear spacing and the number of valves |
| 10.0 | Reliability | 20 % | 2 | Nonfunctional | Relates to availability of system |
| 11.0 | ### Reproducibility | 90 % | 4 | Nonfunctional | Enables precision |
| 12.0 | ### Rep Rate |  |  | Functional |  |
| 12.1 | Min | Single pulse | 5 |  |  |
| 12.2 | Max | 10 Hz | 4 |  |  |
| 13.0 | ### Max leak rate | 5 % | 2 | Nonfunctional | Percentage integrated leakage constitutes of flow during jet pulse, depends on vacuum system |
| 14.0 | Interface Constraints |  |  | Interface |  |
| 14.1 | Fit in existing vacuum chamber | yes | 3 | Interface | Building 71 I’OASIS facility |
| 14.2 | Vacuum operating regime | 1e-3 Torr | 3 | Interface | Vacuum compatibility – relates to pumping capacity of chamber |
| 15.0 | Begin operation | Summer 2003 | 2 | Implementation |  |
| 16.0 | Total Cost | $1 M | 1 | Implementation |  |