MATERIALS FOR RADIOPURITY CONTROL IN NEXT

June 2011

SHIELDING

Lead

Copper

Support structures: Stainless steel

Water containers

Polyethylene?

VESSEL

Titanium

Nuts and bolts: Titanium, with silver plating, polymer coating?

O-ring: PCTFE

Shield ring: copper

Welding: electron beam

At PMT head:

CF flanges: Titanium

Gaskets: silver, tin, PCTFE?, peek?

HV and EL COMPONENTS

Field cage: copper rings, resistors, peek?

High Voltage feedthroughs: metal rod pressed into a plastic tube (Tefzel or FEP), sniffer port

Electroluminiscence grids: stainless steel? mesh

Light tube: Tetratex fixed over a 3M substrate, coated with TPB; Teflon?

DETECTOR COMPONENTS

Energy plane:

PMTs: Hamamatsu R8520-406, R7378A, R1141MOD?

Windows: sapphire (diffusion bonded), quartz (=fused silica)

Tracking plane:

SiPM: 	daughter boards: cuflon coated with TPB

mother boards: PCB?

SiPM Electronics: wires?, front-end, digitizers, multiplexers, optical transceiver, optical links

OTHERS

Adhesives: Araldite, Epoxy…?

Calibration system?

Slow control: HV sensors?

Gas system: 	pipes, valves? Stainless steel

Temperature, Pressure, Flux sensors?